PURPOSE:

To provide instructions for measuring the image density of microfilms using the Densitometer.

OBJECTIVE:

- To measure image density of microfilms per standards.

GENERAL INFORMATION:

Pursuant to State of Arizona Microfilm standards the following ranges are acceptable for microfilm density: Base + fog = 0.00 through 0.16; Background = 0.80 through 1.40.

EVENT DRIVER:

- Measuring image density of microfilms is required.

PROCEDURES:

- Density:

  - The measurement of the microfilm's opacity or degree of gray.

  - Single most critical day to day parameter of a microfilm quality program.

- Variations resulting in low density:

  - Line voltage fluctuation.

  - Lamp aging.

  - Dirty Mirrors.

  - Temperature variation in processing.

  - Changes in ambient light conditions.

  - Variations in film.

  - Colored originals.

  - Oxidized or contaminated chemistry in processor.

- Materials necessary for density control:

  - Densitometer.

  - Automatic exposure controls on planetary cameras.

- Density factors relating to image quality:

  - Contrast – difference between high and low density.

  - Fog – inherent density of film.

- Density measurement:

  - Using a calibrated densitometer, measure the high and low densities of the processed film.

    - High density is the reading obtained from the background or dark portion of the image.

    - Low density is the reading obtained from the clear or base portion of the film.

- Enter these readings in the film log book and note variations beyond + or - 0.30.

- Density notations should be the high density reading followed by the low density reading.  (Example: 1.10/0.04).

POLICIES / RELATED DOCUMENTS:

- Arizona Standards for Permanent Records Media and Storage

OTHER PARTIES INVOLVED:

- N/A

TIME / VOLUME:

- Varied

FREQUENCY:

- Varied

PERFORMANCE MEASURE:

- N/A

SYSTEM ACCESS REQUIRED:

- N/A

REPORTS / FORMS:

- Film Log Book

DEFINITIONS:

- N/A

APPLICABILITY:

This procedure applies to all employees of the Clerk of the Court.

REVISION HISTORY:

| Revision Number | Revision Author | Summary of Changes | Approval Date |
| --- | --- | --- | --- |
| - | N/A | Document Origination | 11/19/2003 |
| A | N/A | N/A | 08/14/2015 |
| B | Larry Urewicz | Reformatted for the CDI | 01/20/2017 |
| C |  |  |  |